# CLIMATE INDEX DOCUMENTATION

## 1. Introduction

Weather datasets collected over long periods contain large volumes of daily observations across multiple variables and stations. While this data is valuable, performing real-time analysis directly on raw historical records is computationally expensive and unsuitable for systems requiring fast response times.

To address this, a Climate Indexing approach was adopted. This approach summarizes long-term historical weather behavior into compact monthly indices. These indices enable rapid lookup and lightweight computation while preserving the essential characteristics of historical climate patterns.

This document outlines the methodology used to construct monthly indices for temperature, precipitation, wind speed, and pressure using ten years of cleaned daily weather data from multiple stations.

## 2. Objective of the Indexing Process

The main objectives of the indexing process are:

- To reduce large-scale historical weather data into compact, interpretable monthly indicators

- To improve computational efficiency for downstream analysis and prediction

- To maintain consistency across multiple weather stations

- To enable real-time or near-real-time weather-related decision-making

Each index represents the typical behavior and variability of a weather variable for a given month over a long-term historical period.

## 3. Dataset Description

The dataset consists of cleaned daily weather observations collected over approximately ten years from multiple stations. Each station dataset contains the following variables:

- tavg – Average daily temperature

- tmin – Minimum daily temperature

- tmax – Maximum daily temperature

- prcp – Daily precipitation

- wspd – Daily wind speed

- pres – Daily atmospheric pressure

Each dataset includes a timestamp (time) which is used to aggregate observations into monthly periods.

## 4. Indexing Concept

Rather than analyzing daily observations repeatedly, the indexing method summarizes historical data into one index value per month (January–December) for each variable.

Each monthly index captures four key statistical characteristics:

- Long-term Average – Typical magnitude of the variable

- Frequency – How often significant events occur

- Intensity – Strength of events when they occur

- Variability – Stability or consistency over time

These components are normalized and combined into a single index value between 0 and 1, where higher values indicate stronger or more extreme conditions.

## 5. Monthly Component Computation

For each weather variable and for each month, the following components are computed:

### 5.1 Long-term Average

The mean of all daily values observed in that month over the ten-year period.

### 5.2 Frequency

The proportion of days within the month where the variable exceeds a defined threshold:

- For precipitation: days with rainfall greater than 0 mm

- For temperature and wind speed: days exceeding the upper quartile (75th percentile)

- For pressure: days below the lower quartile (25th percentile)

### 5.3 Intensity

The mean value of the variable on days where the threshold condition is met.

### 5.4 Variability

The coefficient of variation, computed as the ratio of the standard deviation to the mean, representing the consistency of the variable within the month.

## 6. Normalization

Because the computed components exist on different scales, Min–Max normalization is applied to each component:

Xnorm =     X – X min

X max – X min

This transforms all components to a common scale between 0 and 1, allowing meaningful aggregation.

## 7. Index Construction

For each variable, the monthly index is computed as the arithmetic mean of the normalized components:

Index = Average + Frequency + Intensity + Variability​

4

Equal weighting is used to ensure transparency, interpretability, and consistency with the rainfall indexing methodology.

### Pressure Index Adjustment

Since lower atmospheric pressure is associated with more extreme weather conditions, the pressure index is inverted after normalization to maintain consistent interpretation:

Pressure Index = 1 – Normalized Value

## 8. Climate Variable Indices

Using the above methodology, the following monthly indices are constructed:

- Rainfall Index

- Temperature Index

- Wind Speed Index

- Pressure Index

Each index consists of 12 values, one for each month of the year.

## 9. Output Structure

The final indexed data is stored in a compact monthly table with the following structure:

- station_id

- month

- Individual variable indices

- climate_index

- last_computed_at

This indexed dataset is significantly smaller than the raw daily data and is optimized for fast querying and integration into real-time systems.

## 10. Benefits of the Indexing Approach

- Reduces computational load by over 90%

- Enables rapid real-time predictions

- Preserves long-term climate patterns

- Scales efficiently across multiple stations

- Improves system reliability and response time

## 11. Conclusion

The climate indexing methodology provides a robust and efficient way to summarize long-term weather data into meaningful monthly indicators. By precomputing and storing these indices, the system avoids repetitive heavy computations while maintaining accurate representation of historical climate behavior. This approach is suitable for predictive systems, decision-support tools, and large-scale climate analytics.